Rotational Equilibrium & Torque lab (with photos for take-home version)

This lab demonstrates the “Rotational Equilibrium & Torque” principles we have been studying in the textbook. Complete all six (6) problems below by drawing sketches as needed and showing your work! Discuss any discrepancies between your calculations and the results shown in the photos.

(60 points total)

Key to photos: Mass in g of the meter stick, the center-of-mass of the empty stick, and masses of the various objects used

Scenario 1: Lever with two weights

- (10 points) Use the correct Physics equations to demonstrate why this lever is balanced. Take into account the masses of the 2 weights, the masses of the paper clips, and the slightly off-center ‘center-of-mass’ of the meter stick (refer to center-of-mass photo).

Scenario 2: Lever with three weights

- (10 points) Use the correct Physics equations to demonstrate why this lever is balanced. Take into account the masses of the 3 weights, the masses of the paper clips, and the slightly off-center ‘center-of-mass’ of the meter stick (refer to center-of-mass photo).

Scenario 3: ‘Cantilevered beam’ with two weights and spring balance

- (10 points) Use the correct Physics equations to demonstrate why this cantilevered beam is balanced. Take into account the masses of the 2 weights, the reading on the spring balance, (and don’t forget about the mass of the spring balance itself!... see photo), the masses of the paper clips, and the slightly off-center ‘center-of-mass’ of the meter stick (refer to center-of-mass photo).

Scenario 4: Off-centered ‘Cantilevered beam’ with two weights and spring balance

- (10 points) Use the correct Physics equations to demonstrate why this cantilevered beam is balanced. This is same scenario as above, except the beam is now off-center. The 2 weights are positioned in exactly the same place as above. Re-do your calculations using the off-center positioning.

Scenario 5: Simply-supported beam (or bridge) with one weight

- (10 points) Use the correct Physics equations to validate the two reaction forces FR(A) and FR(B) in connection with this simply-supported beam. Take into account the load, and don’t forget the mass of the meter stick-beam itself! The weight of the pencils was zeroed-out, so don’t add them. Note also the beam is not supported at 0 and 100!

Key to photos: The empty weight of the stick at left and right ends (31.9 g), the position of the 200 g weight, and the fully-loaded weight at left and right ends.

Scenario 6: Simply-supported beam (or bridge) with two weights

- (10 points) Use the correct Physics equations to validate the two reaction forces FR(A) and FR(B) in connection with this simply-supported beam. This is the same scenario as above, but with 2 weights this time.